# ZQZ5041TQZQ6型清障车 (Tow Truck) Technical Specifications

| Parameter | Value |
| --- | --- |
| Model | ZQZ5041TQZQ6型清障车 (Tow Truck) |
| Dimensions | 5995x2380x2330/2280x2200 mm |
| Total Weight | 4495 kg |
| Curb Weight | 2865 kg |
| Passenger Capacity | Not specified |
| Chassis Model | QL1040MVHAY |
| Manufacturer | Qingling Motors Co., Ltd. |
| Axle Number | Not specified |
| Wheelbase | 3360 mm, 3600 mm, 2800 mm |
| Tire Number & Specs | 7.00R16LT 8PR, 6.50R16LT 10PR, 7.00R16LT |
| Fuel Type | Diesel |
| Emission Standard | GB17691-2018 National VI |
| Engine Model | 4KK1N6LB, 4KK1N6SD, 4KB1-TCG60 |
| Engine Power | 120, 132, 130 HP |
| Engine Displacement | 2498 cc, 2499 cc |
| Top Speed | 115 km/h (with 4KB1-TCG60 engine) |
| Special Features | Flatbed and winch, no towing function, optional flip or integrated board styles, two side operation boxes, ABS, and air/hydraulic brake systems. |
| Side Protection Material | Q235, Bolt connection |
| Rear Protection | Q235, Welded connection, dimensions: 100x50 mm, ground clearance: 490 mm |
| Maximum Load on Flatbed | 1500 kg |